traffic_light_sim.py

from tkgpio import TkCircuit configuration = { "width": 400, "height": 300, "leds": [ {"x": 200, "y": 50, "name": "RED", "pin": 17}, {"x": 200, "y": 100, "name": "YELLOW", "pin": 27}, {"x": 200, "y": 150, "name": "GREEN", "pin": 22} ], "buttons": [ {"x": 100, "y": 230, "name": "Pedestrian Crossing", "pin": 11} ], "title": "Traffic Light Simulation" } circuit = TkCircuit(configuration) @circuit.run def main(): import traffic_light if __name__ == "__main__": print("Starting Traffic Light Simulation...")

traffic_light_sim.py

from gpiozero import LED, Button from time import sleep red_light = LED(17) yellow_light = LED(27) green_light = LED(22) pedestrian_button = Button(11) # initial red_light.on() yellow_light.off() green_light.off() auto_mode = True red_time = 5 yellow_time = 2 green_time = 5 pedestrian_requested = False def pedestrian_crossing_request(): global pedestrian_requested print("Pedestrian requested to cross!") pedestrian_requested = True # กดปุ่ม pedestrian_button.when_pressed = pedestrian_crossing_request def traffic_light_sequence(): global pedestrian_requested while True: # ไฟแดง red_light.on() yellow_light.off() green_light.off() print("Red light: Stop") sleep(red_time) # ไม่กด if auto_mode or pedestrian_requested: # ไฟเขียว red_light.off() yellow_light.off() green_light.on() print("Green light: Go") sleep(green_time) # ไฟเหลือง red_light.off() yellow_light.on() green_light.off() print("Yellow light: Prepare to stop") sleep(yellow_time) pedestrian_requested = False try: traffic_light_sequence() except KeyboardInterrupt: red_light.off() yellow_light.off() green_light.off() print("\nTraffic light simulation stopped")