int Green = 2; int Yellow = 3; int Red = 4; void setup() { pinMode(Green, OUTPUT); pinMode(Yellow, OUTPUT); pinMode(Red, OUTPUT); } void loop() { digitalWrite(Red, LOW); delay(1000); digitalWrite(Green, HIGH); delay(5000); digitalWrite(Green, LOW); delay(1000); digitalWrite(Yellow, HIGH); delay(3000); digitalWrite(Yellow, LOW); delay(1000); digitalWrite(Red, HIGH); delay(6000); }