water_alert.py

from gpiozero import Servo, MCP3008, Button, LED, Buzzer from time import sleep import signal print("Water Level Alert System is running...") led_low = LED(17) led_medium = LED(27) led_high = LED(22) servo = Servo(24) buzzer = Buzzer(25) water_sensor = MCP3008(0) def main(): print("System active. Press Ctrl+C to exit.") while True: water_level = water_sensor.value print(f"Water level: {water_level:.2f} ({int(water_level * 100)}%)") # (Low) (0-33%) if water_level >= 0: led_low.on() else: led_low.off() # Medium (34-66%) if water_level > 0.33: led_medium.on() else: led_medium.off() # High (67-100%) if water_level > 0.66: led_high.on() buzzer.on() else: led_high.off() buzzer.off() servo_position = (2 * water_level) - 1 servo.value = servo_position sleep(0.5) if __name__ == "__main__": main()

water_alert_simulation.py

from tkgpio import TkCircuit configuration = { "name": "Water Level Alert System", "width": 700, "height": 400, "leds": [ {"x": 105, "y": 80, "name": "LED1 (Low)", "pin": 17}, {"x": 205, "y": 80, "name": "LED2 (Medium)", "pin": 27}, {"x": 305, "y": 80, "name": "LED3 (High)", "pin": 22} ], "servos": [ {"x": 500, "y": 80, "name": "Water Gate Servo", "pin": 24, "min_angle": -90, "max_angle": 90, "initial_angle": 0} ], "buzzers": [ {"x": 400, "y": 80, "name": "Alert Buzzer", "pin": 25} ], "adc": { "mcp_chip": 3008, "potentiometers": [ {"x": 250, "y": 200, "name": "Water Level Sensor", "channel": 0} ] }, "labels": [ {"x": 15, "y": 25, "text": "Water Level Alert System", "font": ("Arial", 16, "bold")}, {"x": 250, "y": 180, "text": "Water Level Sensor", "font": ("Arial", 10)}, {"x": 105, "y": 60, "text": "Low", "font": ("Arial", 10)}, {"x": 205, "y": 60, "text": "Medium", "font": ("Arial", 10)}, {"x": 305, "y": 60, "text": "High", "font": ("Arial", 10)}, {"x": 500, "y": 60, "text": "Water Gate", "font": ("Arial", 10)}, {"x": 400, "y": 60, "text": "Buzzer", "font": ("Arial", 10)}, {"x": 15, "y": 280, "text": "Low: 0-33% ¦ Medium: 34-66% ¦ High: 67-100%", "font": ("Arial", 10)} ] } circuit = TkCircuit(configuration) @circuit.run def main(): import water_alert water_alert.main()