flood_control.py

from gpiozero import Servo, MCP3008, Button, LED, Buzzer from time import sleep import signal import paho.mqtt.client as mqtt from paho.mqtt.enums import CallbackAPIVersion print("Water Level Alert System is running...") # Hardware Setup led_low = LED(17) led_medium = LED(27) led_high = LED(22) servo = Servo(24) buzzer = Buzzer(25) water_sensor = MCP3008(0) # MQTT Control Variables manual_control = False forced_servo_position = 0.0 # MQTT Callbacks def on_connect(client, userdata, flags, reason_code, properties): print(f"Connected with result code {reason_code}") client.subscribe("water/control") def on_message(client, userdata, msg): global manual_control, forced_servo_position command = msg.payload.decode().upper() if command == "OPEN": manual_control = True forced_servo_position = 1.0 # เปิดสุด print("Force opening water gate") elif command == "CLOSE": manual_control = True forced_servo_position = -1.0 # ปิดสุด print("Force closing water gate") elif command == "AUTO": manual_control = False print("Returning to auto mode") # MQTT Client Setup client = mqtt.Client(callback_api_version=CallbackAPIVersion.VERSION2) client.on_connect = on_connect client.on_message = on_message client.username_pw_set('pk', 'iot1234') client.connect("127.0.0.1", 1883, 60) client.loop_start() def main(): print("System active. Press Ctrl+C to exit.") try: while True: water_level = water_sensor.value print(f"Water level: {water_level:.2f} ({int(water_level * 100)}%)") # คุมไฟ led_low.on() if water_level >= 0 else led_low.off() led_medium.on() if water_level > 0.33 else led_medium.off() if water_level > 0.66: led_high.on() buzzer.on() else: led_high.off() buzzer.off() # คุม Servo if manual_control: servo.value = forced_servo_position else: servo_position = (2 * water_level) - 1 servo.value = servo_position sleep(0.5) except KeyboardInterrupt: client.loop_stop() print("\nSystem stopped") if __name__ == "__main__": main()

flood_control_sim.py

from tkgpio import TkCircuit import threading configuration = { "name": "Water Level Alert System", "width": 700, "height": 400, "leds": [ {"x": 105, "y": 80, "name": "LED1 (Low)", "pin": 17}, {"x": 205, "y": 80, "name": "LED2 (Medium)", "pin": 27}, {"x": 305, "y": 80, "name": "LED3 (High)", "pin": 22} ], "servos": [ {"x": 500, "y": 80, "name": "Water Gate Servo", "pin": 24, "min_angle": -90, "max_angle": 90, "initial_angle": 0} ], "buzzers": [ {"x": 400, "y": 80, "name": "Alert Buzzer", "pin": 25} ], "adc": { "mcp_chip": 3008, "potentiometers": [ {"x": 250, "y": 200, "name": "Water Level Sensor", "channel": 0} ] }, "labels": [ {"x": 15, "y": 25, "text": "Water Level Alert System", "font": ("Arial", 16, "bold")}, {"x": 250, "y": 180, "text": "Water Level Sensor", "font": ("Arial", 10)}, {"x": 105, "y": 60, "text": "Low", "font": ("Arial", 10)}, {"x": 205, "y": 60, "text": "Medium", "font": ("Arial", 10)}, {"x": 305, "y": 60, "text": "High", "font": ("Arial", 10)}, {"x": 500, "y": 60, "text": "Water Gate", "font": ("Arial", 10)}, {"x": 400, "y": 60, "text": "Buzzer", "font": ("Arial", 10)}, {"x": 15, "y": 280, "text": "Low: 0-33% ¦ Medium: 34-66% ¦ High: 67-100%", "font": ("Arial", 10)} ] } circuit = TkCircuit(configuration) @circuit.run def main(): import flood_control flood_control.main()